Appendix-A

1 –VHDL Code for HHandH

--

-- VHDL Entity MSc.HHandH.symbol

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 21:29:39 08/29/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.all;

USE ieee.numeric_std.all;

ENTITY HHandH IS

PORT(

In0  : IN     std_logic  ;

In1  : IN     std_logic  ;

Out0 : OUT    std_logic

);

-- Declarations

END HHandH ;

--

-- VHDL Architecture MSc.HHandH.dataflow

--

-- Created by:

--          by - iscic ISCI (shark)

--          at - 21:55:19 02/05/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

ARCHITECTURE dataflow OF HHandH IS

BEGIN

Out0 <= (In0 and In1);

END dataflow;

2 –VHDL Code for HLandH

--

-- VHDL Entity MSc.HLandH.symbol

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 20:15:42 07/12/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.all;

USE ieee.numeric_std.all;

ENTITY HLandH IS

PORT(

In0  : IN     std_logic  ;

In1  : IN     std_logic  ;

Out0 : OUT    std_logic

);

-- Declarations

END HLandH ;

--

-- VHDL Architecture MSc.HLandH.dataflow

--

-- Created by:

--          by - iscic ISCI (shark)

--          at - 21:55:19 02/05/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

ARCHITECTURE dataflow OF HLandH IS

BEGIN

Out0 <= (In0 and not(In1));

END dataflow;

3 –VHDL Code for HA

--

-- VHDL Entity MSc.HA.symbol

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 19:47:49 07/12/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.all;

USE ieee.numeric_std.all;

ENTITY HA IS

PORT(

A    : IN     std_logic  ;

B    : IN     std_logic  ;

Cout : OUT    std_logic  ;

S    : OUT    std_logic

);

-- Declarations

END HA ;

--

-- VHDL Architecture MSc.HA.dataflow

--

-- Created by:

--          by - iscic ISCI (shark)

--          at - 18:52:51 07/12/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

ARCHITECTURE dataflow OF HA IS

BEGIN

S <= A xor B;

-- sum func-n

Cout <= (A and B);

-- Carry out func-n

END dataflow;

4 –VHDL Code for FA

--

-- VHDL Entity MSc.FA.symbol

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 19:09:28 07/12/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.all;

USE ieee.numeric_std.all;

ENTITY FA IS

PORT(

A    : IN     std_logic  ;

B    : IN     std_logic  ;

Cin  : IN     std_logic  ;

Cout : OUT    std_logic  ;

S    : OUT    std_logic

);

-- Declarations

END FA ;

--

-- VHDL Architecture MSc.FA.dataflow

--

-- Created by:

--          by - iscic ISCI (shark)

--          at - 18:52:51 07/12/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

ARCHITECTURE dataflow OF FA IS

BEGIN

S <= (A and not(B) and not(Cin)) or (not(A) and B and not(Cin)) or (not(A) and not(B) and Cin) or (A and B and Cin);

-- sum func-n

Cout <= (A and B) or (A and Cin) or (B and Cin);

-- Carry out func-n

END dataflow;

5 –VHDL Code for MSBFA

--

-- VHDL Entity MSc.MSBFA.symbol

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 19:44:40 07/12/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.all;

USE ieee.numeric_std.all;

ENTITY MSBFA IS

PORT(

A    : IN     std_logic  ;

B    : IN     std_logic  ;

Cin  : IN     std_logic  ;

S    : OUT    std_logic  ;

Smsb : OUT    std_logic

);

-- Declarations

END MSBFA ;

--

-- VHDL Architecture MSc.MSBFA.dataflow

--

-- Created by:

--          by - iscic ISCI (shark)

--          at - 18:52:51 07/12/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

ARCHITECTURE dataflow OF MSBFA IS

BEGIN

S <= (A and not(B) and not(Cin)) or (not(A) and B and not(Cin)) or (not(A) and not(B) and Cin) or (A and B and Cin);

-- sum func-n

Smsb <= (A and B) or (A and not(Cin)) or (B and not(Cin));

-- Modified Cout func-n to reveal the sign extended sum o/p for signed addition

END dataflow;

6 –VHDL Code for CPAmult3x3

--

-- VHDL Entity MSc.CPAmult3x3.symbol

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 20:55:10 07/27/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.all;

USE ieee.numeric_std.all;

ENTITY CPAmult3x3 IS

PORT(

A       : IN     std_logic_vector (2 DOWNTO 0) ;

B       : IN     std_logic_vector (2 DOWNTO 0) ;

Product : OUT    std_logic_vector (5 DOWNTO 0)

);

-- Declarations

END CPAmult3x3 ;

--

-- VHDL Architecture MSc.CPAmult3x3.struct

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 20:55:13 07/27/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.ALL;

USE ieee.numeric_std.ALL;

LIBRARY MSc;

ARCHITECTURE struct OF CPAmult3x3 IS

-- Architecture declarations

-- Internal signal declarations

SIGNAL A0B0   : std_logic;

SIGNAL A0B1   : std_logic;

SIGNAL A0B2   : std_logic;

SIGNAL A1B0   : std_logic;

SIGNAL A1B1   : std_logic;

SIGNAL A1B2   : std_logic;

SIGNAL A2_B0  : std_logic;

SIGNAL A2_B1  : std_logic;

SIGNAL A2_B2  : std_logic;

SIGNAL Cout   : std_logic;

SIGNAL Cout_1 : std_logic;

SIGNAL Cout_2 : std_logic;

SIGNAL Cout_3 : std_logic;

SIGNAL S      : std_logic;

SIGNAL S_1    : std_logic;

SIGNAL Smsb   : std_logic;

-- Component Declarations

COMPONENT FA

PORT (

A    : IN     std_logic ;

B    : IN     std_logic ;

Cin  : IN     std_logic ;

Cout : OUT    std_logic ;

S    : OUT    std_logic

);

END COMPONENT;

COMPONENT HA

PORT (

A    : IN     std_logic ;

B    : IN     std_logic ;

Cout : OUT    std_logic ;

S    : OUT    std_logic

);

END COMPONENT;

COMPONENT HHandH

PORT (

In0  : IN     std_logic ;

In1  : IN     std_logic ;

Out0 : OUT    std_logic

);

END COMPONENT;

COMPONENT HLandH

PORT (

In0  : IN     std_logic ;

In1  : IN     std_logic ;

Out0 : OUT    std_logic

);

END COMPONENT;

COMPONENT MSBFA

PORT (

A    : IN     std_logic ;

B    : IN     std_logic ;

Cin  : IN     std_logic ;

S    : OUT    std_logic ;

Smsb : OUT    std_logic

);

END COMPONENT;

-- Optional embedded configurations

-- pragma synthesis_off

FOR ALL : FA USE ENTITY MSc.FA(dataflow);

FOR ALL : HA USE ENTITY MSc.HA(dataflow);

FOR ALL : HHandH USE ENTITY MSc.HHandH(dataflow);

FOR ALL : HLandH USE ENTITY MSc.HLandH(dataflow);

FOR ALL : MSBFA USE ENTITY MSc.MSBFA(dataflow);

-- pragma synthesis_on

BEGIN

-- Architecture concurrent statements

-- HDL Embedded Text Block 1 netcon

Product(0)<= A0B0;

-- Instance port mappings.

I0 : FA

PORT MAP (

A    => A1B1,

B    => A0B2,

Cin  => Cout,

Cout => Cout_1,

S    => S

);

I1 : FA

PORT MAP (

A    => S,

B    => A2_B0,

Cin  => A(2),

Cout => Cout_2,

S    => Product(2)

);

I2 : FA

PORT MAP (

A    => S_1,

B    => A2_B1,

Cin  => Cout_2,

Cout => Cout_3,

S    => Product(3)

);

I3 : HA

PORT MAP (

A    => A1B0,

B    => A0B1,

Cout => Cout,

S    => Product(1)

);

I6 : HHandH

PORT MAP (

In0  => B(2),

In1  => A(0),

Out0 => A0B2

);

I7 : HHandH

PORT MAP (

In0  => B(1),

In1  => A(0),

Out0 => A0B1

);

I8 : HHandH

PORT MAP (

In0  => B(0),

In1  => A(0),

Out0 => A0B0

);

I9 : HHandH

PORT MAP (

In0  => B(2),

In1  => A(1),

Out0 => A1B2

);

I10 : HHandH

PORT MAP (

In0  => B(1),

In1  => A(1),

Out0 => A1B1

);

I11 : HHandH

PORT MAP (

In0  => B(0),

In1  => A(1),

Out0 => A1B0

);

I12 : HLandH

PORT MAP (

In0  => A(2),

In1  => B(1),

Out0 => A2_B1

);

I13 : HLandH

PORT MAP (

In0  => A(2),

In1  => B(2),

Out0 => A2_B2

);

I14 : HLandH

PORT MAP (

In0  => A(2),

In1  => B(0),

Out0 => A2_B0

);

I4 : MSBFA

PORT MAP (

A    => A1B2,

B    => A0B2,

Cin  => Cout_1,

S    => S_1,

Smsb => Smsb

);

I5 : MSBFA

PORT MAP (

A    => A2_B2,

B    => Smsb,

Cin  => Cout_3,

S    => Product(4),

Smsb => Product(5)

);

END struct;

7 –VHDL Code for CPAmultNxN

--

-- VHDL Entity MSc.CPAmultNxN.symbol

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 21:28:42 08/29/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.all;

USE ieee.numeric_std.all;

ENTITY CPAmultNxN IS

GENERIC(

N : integer := 8

);

PORT(

A       : IN     std_logic_vector (N-1 DOWNTO 0) ;

B       : IN     std_logic_vector (N-1 DOWNTO 0) ;

Product : OUT    std_logic_vector (2*N-1 DOWNTO 0)

);

-- Declarations

END CPAmultNxN ;

--

-- VHDL Architecture MSc.CPAmultNxN.struct

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 21:28:46 08/29/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.ALL;

USE ieee.numeric_std.ALL;

LIBRARY MSc;

ARCHITECTURE struct OF CPAmultNxN IS

-- Architecture declarations

type bpmatrix is array (natural range N-1 downto 0, natural range N-1 downto 0) of std_logic;

type carrymatrix is array (natural range 2 to N-2, natural range N-1 downto 1) of std_logic;

type sum2darray is array (1 to N-2) of std_logic_vector(N-1 downto 0);

-- Internal signal declarations

SIGNAL bitproducts  : bpmatrix;

SIGNAL carryripple1 : std_logic_vector(N-1 DOWNTO 1);

SIGNAL carryripple2 : std_logic_vector(N-1 DOWNTO 1);

SIGNAL carryripples : carrymatrix;

SIGNAL sums         : sum2darray;

-- Component Declarations

COMPONENT FA

PORT (

A    : IN     std_logic ;

B    : IN     std_logic ;

Cin  : IN     std_logic ;

Cout : OUT    std_logic ;

S    : OUT    std_logic

);

END COMPONENT;

COMPONENT FA1

PORT (

A    : IN     std_logic ;

B    : IN     std_logic ;

Cin  : IN     std_logic ;

Cout : OUT    std_logic ;

S    : OUT    std_logic

);

END COMPONENT;

COMPONENT FA2

PORT (

A    : IN     std_logic ;

B    : IN     std_logic ;

Cin  : IN     std_logic ;

Cout : OUT    std_logic ;

S    : OUT    std_logic

);

END COMPONENT;

COMPONENT FA3

PORT (

A    : IN     std_logic ;

B    : IN     std_logic ;

Cin  : IN     std_logic ;

Cout : OUT    std_logic ;

S    : OUT    std_logic

);

END COMPONENT;

COMPONENT HA

PORT (

A    : IN     std_logic ;

B    : IN     std_logic ;

Cout : OUT    std_logic ;

S    : OUT    std_logic

);

END COMPONENT;

COMPONENT HA1

PORT (

A    : IN     std_logic ;

B    : IN     std_logic ;

Cout : OUT    std_logic ;

S    : OUT    std_logic

);

END COMPONENT;

COMPONENT HHandH

PORT (

In0  : IN     std_logic ;

In1  : IN     std_logic ;

Out0 : OUT    std_logic

);

END COMPONENT;

COMPONENT HLandH

PORT (

In0  : IN     std_logic ;

In1  : IN     std_logic ;

Out0 : OUT    std_logic

);

END COMPONENT;

COMPONENT MSBFA

PORT (

A    : IN     std_logic ;

B    : IN     std_logic ;

Cin  : IN     std_logic ;

S    : OUT    std_logic ;

Smsb : OUT    std_logic

);

END COMPONENT;

COMPONENT MSBFA1

PORT (

A    : IN     std_logic ;

B    : IN     std_logic ;

Cin  : IN     std_logic ;

S    : OUT    std_logic ;

Smsb : OUT    std_logic

);

END COMPONENT;

COMPONENT MSBFA2

PORT (

A    : IN     std_logic ;

B    : IN     std_logic ;

Cin  : IN     std_logic ;

S    : OUT    std_logic ;

Smsb : OUT    std_logic

);

END COMPONENT;

-- Optional embedded configurations

-- pragma synthesis_off

FOR ALL : HHandH USE ENTITY MSc.HHandH(dataflow);

FOR ALL : HLandH USE ENTITY MSc.HLandH(dataflow);

FOR ALL : HA USE ENTITY MSc.HA(dataflow);

FOR ALL : MSBFA USE ENTITY MSc.MSBFA(dataflow);

FOR ALL : FA USE ENTITY MSc.FA(dataflow);

FOR ALL : MSBFA1 USE ENTITY MSc.MSBFA1(dataflow);

FOR ALL : HA1 USE ENTITY MSc.HA1(dataflow);

FOR ALL : FA1 USE ENTITY MSc.FA1(dataflow);

FOR ALL : MSBFA2 USE ENTITY MSc.MSBFA2(dataflow);

FOR ALL : FA3 USE ENTITY MSc.FA3(dataflow);

FOR ALL : FA2 USE ENTITY MSc.FA2(dataflow);

-- pragma synthesis_on

BEGIN

-- Architecture concurrent statements

-- HDL Embedded Text Block 1 netcon

Product(0)<= bitproducts(0,0);

-- Instance port mappings.

g11: FOR row IN 1 TO N-1 GENERATE

g9: IF row=N-1 GENERATE

I7 : MSBFA2

PORT MAP (

S => Product(2*N-2),

Smsb => Product(2*N-1),

Cin => carryripple2(N-1),

A => bitproducts(row,N-1),

B => sums(row-1)(N-1)

);

I9 : FA3

PORT MAP (

Cin => A(N-1),

S => Product(row),

A => bitproducts(row,0),

B => sums(row-1)(0),

Cout => carryripple2(1)

);

g10: FOR col IN 1 TO N-2 GENERATE

I8 : FA2

PORT MAP (

A => bitproducts(row,col),

B => sums(row-1)(col),

Cin => carryripple2(col),

S => product(row+col) ,

Cout => carryripple2(col+1)

);

END GENERATE g10;

END GENERATE g9;

g6: IF row=1 GENERATE

I4 : HA

PORT MAP (

S => Product(1),

Cout => carryripple1(1),

A => bitproducts(row,0),

B => bitproducts(row-1,0+1)

);

I5 : MSBFA

PORT MAP (

Cin => carryripple1(N-1),

A => bitproducts(row,N-1),

B => bitproducts(row-1,N-1),

S => sums(1)(N-2),

Smsb => sums(1)(N-1)

);

g5: FOR col IN 1 TO N-2 GENERATE

I1 : FA

PORT MAP (

A => bitproducts(row,col),

B => bitproducts(row-1,col+1),

Cin => carryripple1(col),

S => sums(1)(col-1),

Cout => carryripple1(col+1)

);

END GENERATE g5;

END GENERATE g6;

g7: IF (row>1)and(row<N-1) GENERATE

I2 : MSBFA1

PORT MAP (

A => bitproducts(row,N-1),

B => sums(row-1)(N-1),

Cin => carryripples(row,N-1),

S => sums(row)(N-2),

Smsb => sums(row)(N-1)

);

I6 : HA1

PORT MAP (

S => product(row),

A => bitproducts(row,0),

B => sums(row-1)(0),

Cout => carryripples(row,1)

);

g8: FOR col IN 1 TO N-2 GENERATE

I3 : FA1

PORT MAP (

A => bitproducts(row,col),

B => sums(row-1)(col),

Cin => carryripples(row,col),

S => sums(row)(col-1),

Cout => carryripples(row,col+1)

);

END GENERATE g8;

END GENERATE g7;

END GENERATE g11;

g1: FOR row IN 0 TO N-1 GENERATE

g2: IF row<N-1 GENERATE

g0: FOR col IN 0 TO N-1 GENERATE

I16 : HHandH

PORT MAP (

In0 => A(row),

In1 => B(col),

Out0 => bitproducts(row,col)

);

END GENERATE g0;

END GENERATE g2;

g3: IF row=N-1 GENERATE

g4: FOR col IN 0 TO N-1 GENERATE

I17 : HLandH

PORT MAP (

In0 => A(row) ,

In1 => B(col),

Out0 => bitproducts(row,col)

);

END GENERATE g4;

END GENERATE g3;

END GENERATE g1;

END struct;

8 –VHDL Code for Signalyzer

--

-- VHDL Entity MSc.Signalyzer.symbol

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 17:53:54 08/29/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.all;

USE ieee.std_logic_arith.all;

ENTITY Signalyzer IS

GENERIC(

width : integer    := 16;

poly  : bit_vector := "10110111101100011"

);

PORT(

ck    : IN     std_logic  ;

clr_l : IN     std_logic  ;

d     : IN     std_logic_vector (width-1 DOWNTO 0) ;

scan  : IN     std_logic  ;

sout  : OUT    std_logic

);

-- Declarations

END Signalyzer ;

--

-- VHDL Architecture DSP.Sig_Analyser.struc

--

-- Created:

--          by - Dik Morling (pascal)

--          at - 21:40 11/06/00

--

-- Generated by Mentor Graphics' Renoir(TM) 99.1 (Build 26)

-- VHDL Architecture MSc.Signalyzer.struc

--

-- Simplified:

--	    by - canturk

--

ARCHITECTURE struc OF Signalyzer IS

-- n-bit parallel-input signature analyser (TDL type feedback)

-- shifts from bit 0 to bit width-1 with async clear

-- scan input disables inputs and lets you clock out signature

-- poly specifys feedback polynomial with the LSB taking the longest path

-- poly='10...1' => char. polynomial = 1.x^width +1.x^(width-1) +...+1.x^0

--      This is supplied as a generic bit_vector of length "width"

-- the serial signature output is taken from the first register in the chain

constant zeros: std_logic_vector(width-1 downto 0):=(others=>'0');

signal qint: std_logic_vector(0 to width-1); -- FF o/p s

-- by this way, the o/p format is better

signal polyv: bit_vector(width downto 0); --internal for poly

BEGIN

polyv<=poly;

SigAn: process (ck, clr_l) --signature analyzer process

variable clrv, sin: X01; -- internal clr & i/p coming from feedback path

begin

clrv:=not To_X01(clr_l); -- Strength strip

if clrv = '1' then

qint <= zeros; -- clear S/R

elsif rising_edge(ck) then

sin:='0';

for i in 0 to width-1 loop --from LSB to MSB of poly

if polyv(i)='1' then

sin:= sin xor qint(width-1-i);

end if;

end loop;

for i in width-1 downto 1 loop

qint(i)<= qint(i-1) xor (not scan and d(i));

end loop;

qint(0)<= sin xor (not scan and d(0));

end if;

end process SigAn;

sout<=qint(0);

END struc;

9 –VHDL Code for MultSign8x8

--

-- VHDL Entity MSc.MultSignNxN.symbol

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 18:47:51 08/28/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.all;

USE ieee.numeric_std.all;

USE ieee.std_logic_arith.all;

ENTITY MultSignNxN IS

GENERIC(

signpoly : bit_vector := "10110111101100011";

Nmult    : integer    := 8

);

PORT(

A       : IN     std_logic_vector (Nmult-1 DOWNTO 0) ;

B       : IN     std_logic_vector (Nmult-1 DOWNTO 0) ;

ck      : IN     std_logic  ;

clr_l   : IN     std_logic  ;

scan    : IN     std_logic  ;

Product : OUT    std_logic_vector (2*Nmult-1 DOWNTO 0) ;

sout    : OUT    std_logic

);

-- Declarations

END MultSignNxN ;

--

-- VHDL Architecture MSc.MultSignNxN.struct

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 18:47:53 08/28/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.ALL;

USE ieee.numeric_std.ALL;

USE ieee.std_logic_arith.ALL;

LIBRARY MSc;

ARCHITECTURE struct OF MultSignNxN IS

-- Architecture declarations

-- Implicit buffer signal declarations

SIGNAL Product_internal : std_logic_vector (2*Nmult-1 DOWNTO 0);

-- Component Declarations

COMPONENT CPAmultNxN

GENERIC (

N : integer := 8

);

PORT (

A       : IN     std_logic_vector (N-1 DOWNTO 0);

B       : IN     std_logic_vector (N-1 DOWNTO 0);

Product : OUT    std_logic_vector (2*N-1 DOWNTO 0)

);

END COMPONENT;

COMPONENT Signalyzer

GENERIC (

width : integer    := 16;

poly  : bit_vector := "10110111101100011"

);

PORT (

ck    : IN     std_logic ;

clr_l : IN     std_logic ;

d     : IN     std_logic_vector (width-1 DOWNTO 0);

scan  : IN     std_logic ;

sout  : OUT    std_logic

);

END COMPONENT;

-- Optional embedded configurations

-- pragma synthesis_off

FOR ALL : CPAmultNxN USE ENTITY MSc.CPAmultNxN(struct);

FOR ALL : Signalyzer USE ENTITY MSc.Signalyzer(struc);

-- pragma synthesis_on

BEGIN

-- Instance port mappings.

I0 : CPAmultNxN

GENERIC MAP (

N => Nmult

)

PORT MAP (

A       => A,

B       => B,

Product => Product_internal

);

I1 : Signalyzer

GENERIC MAP (

width => 2*Nmult,

poly  => signpoly

)

PORT MAP (

ck    => ck,

clr_l => clr_l,

d     => Product_internal,

scan  => scan,

sout  => sout

);

-- Implicit buffered output assignments

Product <= Product_internal;

END struct;

10 –VHDL Code for LFSR

--

-- VHDL Entity MSc.LFSR.symbol

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 21:28:48 08/29/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.all;

USE ieee.std_logic_arith.all;

ENTITY LFSR IS

GENERIC(

width : integer    := 8;

seed  : bit_vector := "01111011";

taps  : bit_vector := "10001101"

);

PORT(

ck    : IN     std_logic  ;

clr_l : IN     std_logic  ;

Q     : OUT    std_logic_vector (1 TO width)

);

-- Declarations

END LFSR ;

--

-- VHDL Architecture MSc.LFSR.mixed

--

-- Created:

--          by - iscic (Shark)

--          at - 11:40 29/08/01

--

ARCHITECTURE struc OF LFSR IS

-- width bit parametrized LFSR (TDL type)

-- taps are specified in generic taps: 1-> tap exists, 0-> no tap

-- taps(i) defines if tap exists from Q(i)

-- structure is: Q(1) -> Q(2) -> .. -> Q(width)--

--                /\   |   (taps)..              |

--     (feedback) || <-(XORs) <- ..     ...   <--

--

-- seed is defined in seed(1 to width) seed(i) is for Q(i)

-- async clr_l resets the SR to seed value!!

--constant zeros: std_logic_vector(width-1 downto 0):=(others=>'0');

signal Qint: std_logic_vector(1 to width); -- internal FF o/p s

signal tapsint: bit_vector(1 to width); -- internal for taps

--required to make the indexing as desired, since vhdl automatically

--indexes taps generic as (0 TO lastindex=width-1)

BEGIN

tapsint <= taps; --assign tapsint(1 to width) <= taps(0 to width-1)

PRBSgenerate: process (ck, clr_l)

--process generating the PRBS seq. (what LFSR does)

variable clrv, Sin: X01; -- internal clr & i/p coming from feedback path

begin

clrv:=not To_X01(clr_l); -- Strength strip??

if clrv = '1' then

Qint <= To_StdLogicVector(seed);

-- Initialize Qint to seed

elsif rising_edge(ck) then

Sin:='0'; -- feedback i/p initialized to 0 for XOR operation

-- i.e. if XNORs were used instead should be initialized to 1

XORloop: for FF in width downto 1 loop --from last FF to 1st

if tapsint(FF) = '1' then -- if a tap exists from that FF

Sin := Sin xor Qint(FF);

-- XOR that FFs o/p, with all previous XORs' result Sin

end if;

end loop XORloop;

-- The result of mod2sum of taps (XORs) is now in Sin

ShiftReg: for FF in width downto 2 loop -- from last ff to 2nd

Qint(FF)<= Qint(FF-1); -- shift values 1 bit right

end loop ShiftReg;

Qint(1)<= Sin; -- Sin from feedback loop is now stored in 1st FF

end if;

end process PRBSgenerate;

Q <= Qint; -- assign actual o/p to internal Qint o/ps concurrently

END struc;

11 –VHDL Code for LFSRMultSignNxN

--

-- VHDL Entity MSc.LFSRMultSignNxN.symbol

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 21:28:24 08/29/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.all;

USE ieee.numeric_std.all;

USE ieee.std_logic_arith.all;

ENTITY LFSRMultSignNxN IS

GENERIC(

signpoly   : bit_vector := "10110111101100011";

Nmult      : integer    := 8;

LFSRtaps   : bit_vector := "10001101";

LFSRseed   : bit_vector := "01111011";

LFSRlength : integer    := 8

);

PORT(

ck      : IN     std_logic  ;

clr_l   : IN     std_logic  ;

scan    : IN     std_logic  ;

Product : OUT    std_logic_vector (2*Nmult-1 DOWNTO 0) ;

sout    : OUT    std_logic

);

-- Declarations

END LFSRMultSignNxN ;

--

-- VHDL Architecture MSc.LFSRMultSignNxN.struct

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 21:28:26 08/29/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.ALL;

USE ieee.numeric_std.ALL;

USE ieee.std_logic_arith.ALL;

LIBRARY MSc;

ARCHITECTURE struct OF LFSRMultSignNxN IS

-- Architecture declarations

-- Internal signal declarations

SIGNAL A : std_logic_vector(Nmult-1 DOWNTO 0);

SIGNAL B : std_logic_vector(Nmult-1 DOWNTO 0);

SIGNAL Q : std_logic_vector(1 TO LFSRlength);

-- Implicit buffer signal declarations

SIGNAL Product_internal : std_logic_vector (2*Nmult-1 DOWNTO 0);

-- Component Declarations

COMPONENT CPAmultNxN

GENERIC (

N : integer := 8

);

PORT (

A       : IN     std_logic_vector (N-1 DOWNTO 0);

B       : IN     std_logic_vector (N-1 DOWNTO 0);

Product : OUT    std_logic_vector (2*N-1 DOWNTO 0)

);

END COMPONENT;

COMPONENT LFSR

GENERIC (

width : integer    := 8;

seed  : bit_vector := "01111011";

taps  : bit_vector := "10001101"

);

PORT (

ck    : IN     std_logic ;

clr_l : IN     std_logic ;

Q     : OUT    std_logic_vector (1 TO width)

);

END COMPONENT;

COMPONENT Signalyzer

GENERIC (

width : integer    := 16;

poly  : bit_vector := "10110111101100011"

);

PORT (

ck    : IN     std_logic ;

clr_l : IN     std_logic ;

d     : IN     std_logic_vector (width-1 DOWNTO 0);

scan  : IN     std_logic ;

sout  : OUT    std_logic

);

END COMPONENT;

-- Optional embedded configurations

-- pragma synthesis_off

FOR ALL : CPAmultNxN USE ENTITY MSc.CPAmultNxN(struct);

FOR ALL : LFSR USE ENTITY MSc.LFSR(struc);

FOR ALL : Signalyzer USE ENTITY MSc.Signalyzer(struc);

-- pragma synthesis_on

BEGIN

-- Architecture concurrent statements

-- HDL Embedded Text Block 1 Bus_Assigner

A <= Q(5 to 8) & Q(5 to 8);

B <= Q(1 to 4) & Q(1 to 4);

-- Instance port mappings.

I0 : CPAmultNxN

GENERIC MAP (

N => Nmult

)

PORT MAP (

A       => A,

B       => B,

Product => Product_internal

);

I2 : LFSR

GENERIC MAP (

width => LFSRlength,

seed  => LFSRseed,

taps  => LFSRtaps

)

PORT MAP (

ck    => ck,

clr_l => clr_l,

Q     => Q

);

I1 : Signalyzer

GENERIC MAP (

width => 2*Nmult,

poly  => signpoly

)

PORT MAP (

ck    => ck,

clr_l => clr_l,

d     => Product_internal,

scan  => scan,

sout  => sout

);

-- Implicit buffered output assignments

Product <= Product_internal;

END struct;

12 –VHDL Code for LFSRMultSignNxN_16

--

-- VHDL Entity MSc.LFSRMultSignNxN_16.symbol

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 20:27:04 08/31/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.all;

USE ieee.numeric_std.all;

USE ieee.std_logic_arith.all;

ENTITY LFSRMultSignNxN_16 IS

GENERIC(

signpoly   : bit_vector := "100010100010101010001010000000101";

Nmult      : integer    := 16;

LFSRtaps   : bit_vector := "10001101";

LFSRseed   : bit_vector := "01111011";

LFSRlength : integer    := 8

);

PORT(

ck      : IN     std_logic  ;

clr_l   : IN     std_logic  ;

scan    : IN     std_logic  ;

Product : OUT    std_logic_vector (2*Nmult-1 DOWNTO 0) ;

sout    : OUT    std_logic

);

-- Declarations

END LFSRMultSignNxN_16 ;

--

-- VHDL Architecture MSc.LFSRMultSignNxN_16.struct

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 20:27:06 08/31/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.ALL;

USE ieee.numeric_std.ALL;

USE ieee.std_logic_arith.ALL;

LIBRARY MSc;

ARCHITECTURE struct OF LFSRMultSignNxN_16 IS

-- Architecture declarations

-- Internal signal declarations

SIGNAL A : std_logic_vector(Nmult-1 DOWNTO 0);

SIGNAL B : std_logic_vector(Nmult-1 DOWNTO 0);

SIGNAL Q : std_logic_vector(1 TO LFSRlength);

-- Implicit buffer signal declarations

SIGNAL Product_internal : std_logic_vector (2*Nmult-1 DOWNTO 0);

-- Component Declarations

COMPONENT CPAmultNxN

GENERIC (

N : integer := 8

);

PORT (

A       : IN     std_logic_vector (N-1 DOWNTO 0);

B       : IN     std_logic_vector (N-1 DOWNTO 0);

Product : OUT    std_logic_vector (2*N-1 DOWNTO 0)

);

END COMPONENT;

COMPONENT LFSR

GENERIC (

width : integer    := 8;

seed  : bit_vector := "01111011";

taps  : bit_vector := "10001101"

);

PORT (

ck    : IN     std_logic ;

clr_l : IN     std_logic ;

Q     : OUT    std_logic_vector (1 TO width)

);

END COMPONENT;

COMPONENT Signalyzer

GENERIC (

width : integer    := 16;

poly  : bit_vector := "10110111101100011"

);

PORT (

ck    : IN     std_logic ;

clr_l : IN     std_logic ;

d     : IN     std_logic_vector (width-1 DOWNTO 0);

scan  : IN     std_logic ;

sout  : OUT    std_logic

);

END COMPONENT;

-- Optional embedded configurations

-- pragma synthesis_off

FOR ALL : CPAmultNxN USE ENTITY MSc.CPAmultNxN(struct);

FOR ALL : LFSR USE ENTITY MSc.LFSR(struc);

FOR ALL : Signalyzer USE ENTITY MSc.Signalyzer(struc);

-- pragma synthesis_on

BEGIN

-- Architecture concurrent statements

-- HDL Embedded Text Block 1 Bus_Assigner

A <= Q(5 to 8) & Q(5 to 8) & Q(5 to 8) & Q(5 to 8);

B <= Q(1 to 4) & Q(1 to 4) & Q(1 to 4) & Q(1 to 4);

-- Instance port mappings.

I0 : CPAmultNxN

GENERIC MAP (

N => Nmult

)

PORT MAP (

A       => A,

B       => B,

Product => Product_internal

);

I2 : LFSR

GENERIC MAP (

width => LFSRlength,

seed  => LFSRseed,

taps  => LFSRtaps

)

PORT MAP (

ck    => ck,

clr_l => clr_l,

Q     => Q

);

I1 : Signalyzer

GENERIC MAP (

width => 2*Nmult,

poly  => signpoly

)

PORT MAP (

ck    => ck,

clr_l => clr_l,

d     => Product_internal,

scan  => scan,

sout  => sout

);

-- Implicit buffered output assignments

Product <= Product_internal;

END struct;

13 –VHDL Code for LFSRMultSignNxN_32

--

-- VHDL Entity MSc.LFSRMultSignNxN_32.symbol

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 16:06:43 09/01/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.all;

USE ieee.numeric_std.all;

USE ieee.std_logic_arith.all;

ENTITY LFSRMultSignNxN_32 IS

GENERIC(

signpoly   : bit_vector := "10000000100010000000100010001000100000001000100000000000000010001";

Nmult      : integer    := 32;

LFSRtaps   : bit_vector := "10001101";

LFSRseed   : bit_vector := "01111011";

LFSRlength : integer    := 8

);

PORT(

ck      : IN     std_logic  ;

clr_l   : IN     std_logic  ;

scan    : IN     std_logic  ;

Product : OUT    std_logic_vector (2*Nmult-1 DOWNTO 0) ;

sout    : OUT    std_logic

);

-- Declarations

END LFSRMultSignNxN_32 ;

--

-- VHDL Architecture MSc.LFSRMultSignNxN_32.struct

--

-- Created:

--          by - iscic.enduser (shark)

--          at - 16:06:45 09/01/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.ALL;

USE ieee.numeric_std.ALL;

USE ieee.std_logic_arith.ALL;

LIBRARY MSc;

ARCHITECTURE struct OF LFSRMultSignNxN_32 IS

-- Architecture declarations

-- Internal signal declarations

SIGNAL A : std_logic_vector(Nmult-1 DOWNTO 0);

SIGNAL B : std_logic_vector(Nmult-1 DOWNTO 0);

SIGNAL Q : std_logic_vector(1 TO LFSRlength);

-- Implicit buffer signal declarations

SIGNAL Product_internal : std_logic_vector (2*Nmult-1 DOWNTO 0);

-- Component Declarations

COMPONENT CPAmultNxN

GENERIC (

N : integer := 8

);

PORT (

A       : IN     std_logic_vector (N-1 DOWNTO 0);

B       : IN     std_logic_vector (N-1 DOWNTO 0);

Product : OUT    std_logic_vector (2*N-1 DOWNTO 0)

);

END COMPONENT;

COMPONENT LFSR

GENERIC (

width : integer    := 8;

seed  : bit_vector := "01111011";

taps  : bit_vector := "10001101"

);

PORT (

ck    : IN     std_logic ;

clr_l : IN     std_logic ;

Q     : OUT    std_logic_vector (1 TO width)

);

END COMPONENT;

COMPONENT Signalyzer

GENERIC (

width : integer    := 16;

poly  : bit_vector := "10110111101100011"

);

PORT (

ck    : IN     std_logic ;

clr_l : IN     std_logic ;

d     : IN     std_logic_vector (width-1 DOWNTO 0);

scan  : IN     std_logic ;

sout  : OUT    std_logic

);

END COMPONENT;

-- Optional embedded configurations

-- pragma synthesis_off

FOR ALL : CPAmultNxN USE ENTITY MSc.CPAmultNxN(struct);

FOR ALL : LFSR USE ENTITY MSc.LFSR(struc);

FOR ALL : Signalyzer USE ENTITY MSc.Signalyzer(struc);

-- pragma synthesis_on

BEGIN

-- Architecture concurrent statements

-- HDL Embedded Text Block 1 Bus_Assigner

A <= Q(5 to 8) & Q(5 to 8) & Q(5 to 8) & Q(5 to 8) & Q(5 to 8) & Q(5 to 8) & Q(5 to 8) & Q(5 to 8);

B <= Q(1 to 4) & Q(1 to 4) & Q(1 to 4) & Q(1 to 4) & Q(1 to 4) & Q(1 to 4) & Q(1 to 4) & Q(1 to 4);

-- Instance port mappings.

I0 : CPAmultNxN

GENERIC MAP (

N => Nmult

)

PORT MAP (

A       => A,

B       => B,

Product => Product_internal

);

I2 : LFSR

GENERIC MAP (

width => LFSRlength,

seed  => LFSRseed,

taps  => LFSRtaps

)

PORT MAP (

ck    => ck,

clr_l => clr_l,

Q     => Q

);

I1 : Signalyzer

GENERIC MAP (

width => 2*Nmult,

poly  => signpoly

)

PORT MAP (

ck    => ck,

clr_l => clr_l,

d     => Product_internal,

scan  => scan,

sout  => sout

);

-- Implicit buffered output assignments

Product <= Product_internal;

END struct;

14 –VHDL Code for LFSRMultSignNxN_24

--

-- VHDL Entity MSc.LFSRMultSignNxN_24.symbol

--

-- Created:

--          by - iscic.enduser (bowhead)

--          at - 13:09:58 09/05/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.all;

USE ieee.numeric_std.all;

USE ieee.std_logic_arith.all;

ENTITY LFSRMultSignNxN_24 IS

GENERIC(

signpoly   : bit_vector := "1011111000110000101010011010100000110111011101111";

Nmult      : integer    := 24;

LFSRtaps   : bit_vector := "10001101";

LFSRseed   : bit_vector := "01111011";

LFSRlength : integer    := 8

);

PORT(

ck      : IN     std_logic  ;

clr_l   : IN     std_logic  ;

scan    : IN     std_logic  ;

Product : OUT    std_logic_vector (2*Nmult-1 DOWNTO 0) ;

sout    : OUT    std_logic

);

-- Declarations

END LFSRMultSignNxN_24 ;

--

-- VHDL Architecture MSc.LFSRMultSignNxN_24.struct

--

-- Created:

--          by - iscic.enduser (bowhead)

--          at - 13:10:00 09/05/01

--

-- Generated by Mentor Graphics' Renoir(TM) 99.6 (Build 28)

--

LIBRARY ieee ;

USE ieee.std_logic_1164.ALL;

USE ieee.numeric_std.ALL;

USE ieee.std_logic_arith.ALL;

LIBRARY MSc;

ARCHITECTURE struct OF LFSRMultSignNxN_24 IS

-- Architecture declarations

-- Internal signal declarations

SIGNAL A : std_logic_vector(Nmult-1 DOWNTO 0);

SIGNAL B : std_logic_vector(Nmult-1 DOWNTO 0);

SIGNAL Q : std_logic_vector(1 TO LFSRlength);

-- Implicit buffer signal declarations

SIGNAL Product_internal : std_logic_vector (2*Nmult-1 DOWNTO 0);

-- Component Declarations

COMPONENT CPAmultNxN

GENERIC (

N : integer := 8

);

PORT (

A       : IN     std_logic_vector (N-1 DOWNTO 0);

B       : IN     std_logic_vector (N-1 DOWNTO 0);

Product : OUT    std_logic_vector (2*N-1 DOWNTO 0)

);

END COMPONENT;

COMPONENT LFSR

GENERIC (

width : integer    := 8;

seed  : bit_vector := "01111011";

taps  : bit_vector := "10001101"

);

PORT (

ck    : IN     std_logic ;

clr_l : IN     std_logic ;

Q     : OUT    std_logic_vector (1 TO width)

);

END COMPONENT;

COMPONENT Signalyzer

GENERIC (

width : integer    := 16;

poly  : bit_vector := "10110111101100011"

);

PORT (

ck    : IN     std_logic ;

clr_l : IN     std_logic ;

d     : IN     std_logic_vector (width-1 DOWNTO 0);

scan  : IN     std_logic ;

sout  : OUT    std_logic

);

END COMPONENT;

-- Optional embedded configurations

-- pragma synthesis_off

FOR ALL : CPAmultNxN USE ENTITY MSc.CPAmultNxN(struct);

FOR ALL : LFSR USE ENTITY MSc.LFSR(struc);

FOR ALL : Signalyzer USE ENTITY MSc.Signalyzer(struc);

-- pragma synthesis_on

BEGIN

-- Architecture concurrent statements

-- HDL Embedded Text Block 1 Bus_Assigner

A <= Q(5 to 8) & Q(5 to 8) & Q(5 to 8) & Q(5 to 8) & Q(5 to 8) & Q(5 to 8);

B <= Q(1 to 4) & Q(1 to 4) & Q(1 to 4) & Q(1 to 4) & Q(1 to 4) & Q(1 to 4);

-- Instance port mappings.

I0 : CPAmultNxN

GENERIC MAP (

N => Nmult

)

PORT MAP (

A       => A,

B       => B,

Product => Product_internal

);

I2 : LFSR

GENERIC MAP (

width => LFSRlength,

seed  => LFSRseed,

taps  => LFSRtaps

)

PORT MAP (

ck    => ck,

clr_l => clr_l,

Q     => Q

);

I1 : Signalyzer

GENERIC MAP (

width => 2*Nmult,

poly  => signpoly

)

PORT MAP (

ck    => ck,

clr_l => clr_l,

d     => Product_internal,

scan  => scan,

sout  => sout

);

-- Implicit buffered output assignments

Product <= Product_internal;

END struct;